# Street network and layout design

## Objectives

- To integrate precinct and network planning with street design to ensure they are coordinated and working together to deliver better street networks and encourage mode shift from private vehicles to walking, cycling, and public transport;

- To emphasise the close coordination required between the street type choice, its detailed design, and the subdivision of lots or cadastral “Place” determinants, encouraging consideration beyond public domain ‘things’ within the street reserve like trees, to more integrated design along the street/lot interface;

- To create interesting and attractive streetscapes that contribute to the identity and amenity of every neighbourhood;

- To establish a legible network and interconnected hierarchy of self-explaining slow streetscape environments that provide safe, convenient, and clear access within and beyond the Precinct;

- To create safe, accessible, comfortable and attractive public transport, walking, and cycling networks;

- To assist in managing the environmental impacts of urban development including soil salinity, micro-climate effects, stormwater, and the retention of mature trees and significant vegetation;

- To ensure public safety from criminal elements by considering the NSW Police ‘Safer by Design’ or ‘Crime Prevention Through Environmental Design’ principles and protocols;

- To facilitate energy efficient lot and building orientation;

## Controls

### Controls for all street types

- The design and construction of streets is to be consistent with the relevant typical designs in Figure X to Figure X, the Western Sydney Street Design Guidelines, Council’s Engineering Specifications, and Austroads (where applicable).

- All street cross-sections illustrate minimum requirements. In particular circumstances, widths may need to be modified to respond to site-specific conditions, such as topography and the retention of remnant vegetation, or to incorporate additional design requirements such as water sensitive urban design measures.

- All streets which form part of a bus route identified by Transport for NSW are to have at least one travel lane in each direction with a minimum width of 3.5 metres, suitable for buses. Lanes which are not adjacent to a kerb may be 3.2m wide. Intersections on bus routes are to be designed to accommodate bus manoeuvrability.

- Alternative street designs for local streets and access ways may be permitted on a case-by-case basis if they preserve the functional objectives and requirements of the design standards presented herein and within the Western Sydney Street Design Guidelines, Council’s Engineering Specifications, and Austroads (where applicable).

- The locations and alignments of all streets are to be generally in accordance with the locations shown on the <insert cross reference to appropriate precinct street hierarchy figure if relevant>.

- Where any variation to the residential street network indicated in the <insert cross reference to appropriate precinct street hierarchy figure if relevant> is proposed, the alternative street network is to be designed to:

  - align with the intent and functional requirements of the Western Sydney Street Design Guidelines;

  - not detrimentally impact on access to adjoining properties;

  - provide for the management of stormwater to drain to Council’s trunk drainage network, without negative impacts on other properties;

  - not impede the orderly development of adjoining properties in accordance with the relevant Precinct Plan and this Development Control Plan;

  - not restrict the ability to provide water, sewer, electricity and other essential services to the development or to development on adjoining properties;

  - create a permeable network that is based on a modified grid system;

  - encourage walking and cycling;

  - maximise connectivity between residential areas and community facilities, open space, and centres;

  - take account of topography and site drainage, and accommodate the retention of mature trees and significant vegetation;

  - optimise solar access opportunities for dwellings and other buildings;

  - provide frontage to and maximise surveillance of open space and drainage lands;

  - provide views and vistas to landscape features and visual connections to nodal points and centres;

  - maximise the effectiveness of water sensitive urban design measures;

  - ensure that noise impacts from major roads are considered and are able to be effectively mitigated without the use of noise walls;

  - minimise the use of cul-de-sacs; and

  - comply with the requirements of Planning for Bushfire Protection 2006.

- Where streets are adjacent to public open space or drainage land, or adjacent to arterial, sub-arterial or transit boulevards, the verge width on the side adjacent to the open space, drainage land or major road may, in certain circumstances, be reduced to a minimum of 1m, subject to:

  - appropriate arrangements for the provision of public utilities;

  - provision of appropriate walking and cycling facilities within the adjacent open space, drainage land, or major road;

  - compliance with road safety requirements; and

  - acoustic attenuation, bushfire asset protection zone, and riparian corridor requirements.

- Where streets are located as per control 10 above, the parking bays or parking lanes may be provided on one side only providing the applicant can demonstrate to Council’s satisfaction that the street will operate safely and effectively. Street trees are still to be provided within both verges.

- Where streets are proposed as part of an application for subdivision that are located adjacent to public recreation land, drainage land, community facilities or schools, the applicant will be responsible for construction of the full width of the street.

- Private streets are to be designed and constructed in accordance with the Western Sydney Street Design Guidelines and Council’s Engineering Specifications. Details must be shown on the engineering design plans and must be submitted prior to the issue of the subdivision certificate.

- The design of streets is to take account of existing mature trees and significant vegetation.

### Destination High Streets

Destination High Streets are to adhere to the following performance criteria:

| Road reserve | ≥20m |
| --- | --- |
| Typical AADT | ≤10,000 |
| Freight access | Service and delivery vehicles |
| Posted speed | 30-40km/h |
| Design speed | Same as posted speed |
| Design vehicle | B85 car |
| Check vehicle | Single unit truck/bus (12.5m) |
| Bus capable | Yes |
| Vehicle travel lanes | 2x 3.2m (+ 0.3m buffer for bus capability) |
| Parking provision | Varies |
| Parking arrangement | 2.3m indented parking bays |
| Walking | ≥3.0m footpaths on both sides |
| Cycling | Within travel lane |
| Traffic calming | ≤80m spacings |
| WSUD | Passive irrigation (low flow), Swale (high flow) |
| Tree canopy cover | 70% (nom.) – refer NSW Greener Neighbourhoods Guide |
| Permeability | ≥35% |

Figure X – Destination High Street typical cross section

Figure X – Destination High Street typical plan

### Enterprise Streets

Destination High Streets are to adhere to the following performance criteria:

| Road reserve | ≥ 21.5m |
| --- | --- |
| Typical AADT | 1,000 - 4,000 |
| Freight access | Service and delivery vehicles |
| Posted speed | 40-50km/h |
| Design speed | Same as posted speed |
| Design vehicle | 19m semi-trailer |
| Check vehicle | 19m semi-trailer |
| Bus capable | Yes |
| Vehicle travel lanes | 2 x 3.2m (+ 0.3m buffer for bus capability) |
| Parking provision | Varies |
| Parking arrangement | 2.3 - 3.0m indented parking bays (widen if needed for design vehicle such as freight/delivery vehicles to park without encroaching in travel lane) |
| Walking | 1x ≥1.5m footpath + 1x ≥3.0m shared path |
| Cycling | 1x ≥3.0m shared path |
| Traffic calming | ≤80m spacings |
| WSUD | Passive irrigation (low flow), Swale (high flow) |
| Tree canopy cover | 60% (nom.) – refer NSW Greener Neighbourhoods Guide |
| Permeability | ≥35% |

Figure X – Enterprise Street typical cross section

Figure X – Enterprise Street typical plan

### Urban Centre Streets

Urban Centre Streets are to adhere to the following performance criteria:

| Road reserve | ≥ 21.5m |
| --- | --- |
| Typical AADT | 1,000 - 4,000 |
| Freight access | Service and delivery vehicles |
| Posted speed | 40-50km/h |
| Design speed | Same as posted speed |
| Design vehicle | 8.8m service vehicle |
| Check vehicle | 8.8m service vehicle 12.5m bus (if on bus route) |
| Bus capable | Yes |
| Vehicle travel lanes | 2x 3.2m (+ 0.3m buffer for bus capability) |
| Parking provision | Varies |
| Parking arrangement | 2.3m indented parking bays |
| Walking | 1x ≥1.5m footpath + 1x ≥3.0m shared path |
| Cycling | ≥2.5m separated cycleway or 2x ≥1.5m unidirectional cycleways |
| Traffic calming | ≤80m spacings |
| WSUD | Passive irrigation (low flow), Swale (high flow) |
| Tree canopy cover | 70% (nom.) – refer NSW Greener Neighbourhoods Guide |
| Permeability | ≥35% |

Figure X – Urban Centre Street typical cross section

Figure X – Urban Centre Street typical plan

### Connector Streets

Connector Streets are to adhere to the following performance criteria:

| Road reserve | ≥21.5m |
| --- | --- |
| Typical AADT | 2,000 - 5,000 |
| Freight access | Service and delivery vehicles |
| Posted speed | 40-50km/h |
| Design speed | Same as posted speed |
| Design vehicle | 8.8m service vehicle, 12.5m bus (if on bus route) |
| Check vehicle | 12.5m bus/truck |
| Bus capable | Yes |
| Vehicle travel lanes | 2x 3.2m (+ 0.3m buffer for bus capability) |
| Parking provision | Varies |
| Parking arrangement | 2.3m indented parking bays |
| Walking | ≥1.5m footpaths on both sides; or 1x ≥1.5m footpath + 1x ≥3.0m shared path |
| Cycling | ≥2.5m separated cycleway; or 1x ≥3.0m shared path |
| Traffic calming | ≤80m spacings |
| WSUD | Passive irrigation (low flow), Swale (high flow) |
| Tree canopy cover | 70% (nom.) – refer NSW Greener Neighbourhoods Guide |
| Permeability | ≥35% |

Figure X – Connector Street (with separated cycleway) typical cross section

Figure X – Connector Street (with separated cycleway) typical plan

Figure X – Connector Street (with shared path) typical cross section

Figure X – Connector Street (with shared path) typical plan

### Neighbourhood Streets

Neighbourhood Streets are to adhere to the following performance criteria:

| Road reserve | ≥18m |
| --- | --- |
| Typical AADT | 500 - 2,000 |
| Posted speed | 30-40 km/h |
| Design speed | Same as posted speed |
| Design vehicle | 5.3m passenger car |
| Check vehicle | 8.8m service vehicle, 12.5m bus (if on bus route) |
| Bus capable | Varies |
| Vehicle travel lanes | 2 x 3.0m (3.2m on bus routes) |
| Parking provision | ≤0.75 per lot |
| Parking arrangement | 2.1m indented parking bays |
| Walking | ≥1.5m footpaths on both sides |
| Cycling | Quietway - refer TfNSW Cycleway Design Toolbox |
| Traffic calming | ≤80m spacings |
| WSUD | Passive irrigation (low flow), Swale (high flow) |
| Tree canopy cover | 70% (nom.) – refer NSW Greener Neighbourhoods Guide |
| Permeability | ≥35% |

Figure X – Neighbourhood Street typical cross section

Figure X – Neighbourhood Street typical plan

### Yield Streets

Yield Streets are to adhere to the following performance criteria:

| Road reserve | ≥16m |
| --- | --- |
| Typical AADT | 500 - 2,000 |
| Posted speed | 20-40km/h |
| Design speed | Same as posted speed |
| Design vehicle | 5.3m passenger car |
| Check vehicle | 8.8m service vehicle |
| Bus capable | No |
| Vehicle travel lanes | =7.6m kerb-to-kerb two-way carriageway |
| Parking provision | ≤0.75 per lot |
| Parking arrangement | Kerbside within carriageway to enforce yield for vehicles (no indented bays) |
| Walking | ≥1.5m footpaths on both sides |
| Cycling | Quietway - refer TfNSW Cycleway Design Toolbox |
| Traffic calming | ≤80m spacings |
| WSUD | Passive irrigation (low flow), Swale (high flow) |
| Tree canopy cover | 70% (nom.) – refer NSW Greener Neighbourhoods Guide |
| Permeability | ≥35% |

Figure X – Yield Street typical cross section

Figure X – Yield Street typical plan

### Residential Ways

Residential Ways are to adhere to the following performance criteria:

| Road reserve | ≥14m |
| --- | --- |
| Typical lots served | ≈10-15 |
| Typical AADT | 100 - 1,000 |
| Posted speed | 10-20km/h |
| Design speed | Same as posted speed |
| Design vehicle | 5.3m passenger car |
| Check vehicle | 8.8m service vehicle |
| Bus capable | No |
| Vehicle travel lanes | 5.6m one or two-way shared zone |
| Parking provision | ≤0.5 per lot |
| Parking arrangement | ≤2.4m indented parking bays (with ‘parking in bays only’ signage) |
| Walking | 1.2m footpath and within shared zone |
| Cycling | Within shared zone |
| Traffic calming | ≤80m spacings |
| WSUD | Passive irrigation (low flow), Swale (high flow) |
| Tree canopy cover | 70% (nom.) – refer NSW Greener Neighbourhoods Guide |
| Permeability | ≥35% |

Figure X – Residential Way typical cross section

Figure X – Residential Way typical cross section

### Residential Lanes

Residential Lanes are to adhere to the following performance criteria:

| Road reserve | ≤8m |
| --- | --- |
| Typical lots served | 8-12m |
| Typical AADT | N/A |
| Posted speed | 10km/h |
| Design speed | 10km/h |
| Design vehicle | 5.3m passenger car |
| Check vehicle | 8.8m service vehicle |
| Bus capable | No |
| Vehicle travel lanes | ≥5.5m shared zone |
| Parking provision | Nil |
| Parking arrangement | N/A |
| Walking | Shared zone |
| Cycling | Two-way cycling within shared zone |
| Traffic calming | Shared zone |
| WSUD | Passive irrigation (low flow), Swale (high flow) |
| Tree canopy cover | 70% (nom.) – refer NSW Greener Neighbourhoods Guide |
| Permeability | ≥20% |

Figure X – Residential Lane typical cross section

Figure X – Residential Lane typical plan

### Civic Lanes

Civic Lanes are to adhere to the following performance criteria:

| Road reserve | ≥7m |
| --- | --- |
| Posted speed | 10km/h |
| Design speed | 10km/h |
| Design vehicle | 5.3m passenger car |
| Check vehicle | 8.8m service vehicle |
| Bus capable | No |
| Vehicle travel lanes | One-way shared zone |
| Parking provision | As required |
| Parking arrangement | Interspersed with street trees |
| Walking | Shared zone |
| Cycling | Two-way cycling within shared zone |
| Traffic calming | Shared zone |
| WSUD | Passive irrigation (low flow), Swale (high flow) |
| Tree canopy cover | 40% (nom.) – refer NSW Greener Neighbourhoods Guide |

Figure X – Civic Lane typical cross section

Figure X – Civic Lane typical plan

### Transit Malls

Transit Malls are to adhere to the following performance criteria:

| Road reserve | ≥18m |
| --- | --- |
| Typical AADT | Vehicle restricted |
| Freight access | Service and emergency vehicles (managed) |
| Posted speed | 10 - 20km/hr |
| Design speed | Same as posted speed |
| Design vehicle | Largest bus operating in space |
| Check vehicle | Same as design vehicle |
| Bus capable | Yes |
| Vehicle travel lanes | 2x 3.5m (for buses) |
| Parking provision | N/A |
| Parking arrangement | N/A |
| Walking | ≥3.0m footpaths on both sides |
| Cycling | Within travel lane (excluding light rail track zone) |
| Traffic calming | N/A |
| WSUD | Passive irrigation (low flow) |
| Tree canopy cover | 70% (nom.) – refer NSW Greener Neighbourhoods Guide |
| Permeability | N/A |

Figure X – Transit Mall typical cross section

Figure X – Transit Mall typical plan